Assumption

- Board width W: 0.70 m

- Board length L: 1.30 m

- Ball diameter: 0.10 m

- Camera height above board Z: 3.0 m

- Goal: resolution high enough to reliably detect

  - scoring holes (≈10–12 cm)

  - balls (10 cm)

calculation

For vision tracking

typically needs at least 5–10 pixels across the smallest feature.

For reliable tracking of balls, recommend 12–16 pixels

Aim diameter resolution 15px

4k camera not necessary

Want 1080p (1920 × 1080 px) but high frame rate, for low latency, better tracking

S = min(1920 / 0.7, 1080/1.3) = 830.769 px/m

Scalling board size

Wpx = 830.769 * 0.7 = 581.54 px

Lpx = 830.769 * 1.3 = 1080px (fills vertically)

Focal length fpx = 830.769 * 3 = 2492.31px

Pixels correspond to ball: 0.1 * 830.769 = 83.08 px

10cm hole: 83.08px, 12 cm hole: 99.70px

Convert fpx into focal length (mm)

Using usb camera/ smartphone camera

Usb camera: 1/2.3" → ~ 6.3 mm width (common)

Phone camera: 1/2.55" → ~ 5.0 mm width (common)

Using usb camera: fmm = 8.17mm

- focal length 8.2mm

Using phone camera sensor: fmm = 6.49 mm

- focal length 6.5mm

summary: 3m height, 1080p camera, if usb 8.2mm focal length

Frame Rate

Assuming 1-3 seconds

30fps is enough, smooth trajectory

Frames per event (1–3 s): 30–180

the ball moves ~0.03 m per frame (if 1 m/s)

~8 pixels per frame (0.03 m × 1920 pixels / 1 m).

SUMMARY

Camera: 1080p (1920×1080), 30–60 FPS, mounted 3.0 m from the board.

- Detect ball presence reliably in frame (per-frame detection).

  - Target: detection precision ≥ 0.98, recall ≥ 0.98 in nominal conditions. (ROI threshold)

- Determine collision (ball hits pedal/board) vs passes by.

  - Target: classification accuracy ≥ 0.98, false collision rate ≤ 1%.

- Robustness: maintain targets across speeds, lighting, multiple balls, minor camera misalignment.

Data collection

Total clips: 300 (2-4 s each)

Normal: 140 clips

Edge case/ Stress: 160 clips (lighting changes, multiple balls, very fast throws, occlusions)

Each clip should include at least 1–2 collisions.

Data Distribution

- Ball speed: slow (0.5–1 m/s) 20%; medium (1–3 m/s) 50%; fast (3–6 m/s) 20%; extremes 10%.

- Trajectories: straight center, diagonal, bounce before pedal.

- Outcomes: collision (50% of clips), pass/no collision (35%), ambiguous (15%).

- Multi-ball: 10% of clips contain 2 simultaneous balls; 5% contain 3 or more.

- Background motion: Assume to be same

- Lighting: Assume to be same

Label each clip

70% training, 10-15% validating, 10-15% testing

I could use cross validation

Collision vs passing through

If the ball stays inside the ROI for at least one frame, consider it a collision.

If immediately leave, passing by

collision if ball center enters ROI and overlaps ROI by >= 50% of its diameter for more consecutive frame

background subtraction, thresholding

feed in simple neural network

prediction base on velocity

next step

find the camera

data collection (how to mount the camera, what computer

what is the experiment,

go pro, mounting device